QualiHive Bluetooth Connection Guide

How the ESP32 and the QualiHive app talk: sensor readings in, pump commands out

October 30, 2026  ·  Replaces the October 29, 2026 guide

# 1. Summary

- The ESP32 is a Bluetooth Low Energy (BLE) peripheral. The phone running QualiHive connects to it.

- The ESP32 uses the Nordic UART Service. It has two characteristics, one for each direction.

- ESP32 to phone: sensor readings, sent as notifications on the TX characteristic.

- Phone to ESP32: pump commands, written to the RX characteristic.

- Every message, in both directions, is one line of JSON text ending in a newline (\n).

- The phone runs the pumps. It can run a timed cycle — pump 1, 2, 3 and 4 in turn, each for the time set in the app — and the user can switch any pump on or off by hand, at any time, even in the middle of a cycle. The ESP32 switches each pump on and off when told, and switches a pump off by itself when its time is up.

- No pairing and no PIN. The user connects from inside the app.

# 2. What changed since the October 29 guide

- Pumps are independent. pump_on now switches on only the pump it names and leaves every other pump as it is, so several pumps can run at once. Firmware built to the October 29 guide switches the other pumps off and must be updated (sections 6.2 and 6.3).

- pump_off can name a pump: {"type":"pump_off","pump":2} switches only pump 2 off. Without pump it still switches every pump off (section 6.2).

- Each pump has its own timer. The duration_s rule now applies to every pump separately, and duration_s can be anything from 1 to 7200 (sections 6.2 and 6.3).

- Manual switches in the app. Each pump has an on/off switch on Home that works at any time, including in the middle of a timed cycle. A pump switched on by hand is sent duration_s 7200 (2 hours) as a safety limit (section 6.5).

- The cycle now sends pump_off for the pump it is leaving before pump_on for the next one, and only pump_off for pump 4 when it finishes (section 6.5).

- With several pumps running, send the stage of the highest-numbered one (section 6.4).

- Moisture is gone. The app no longer reads or grades moisture: remove moisture_percent from the firmware. If it is still sent, the app ignores it (sections 5 and 7).

- Pumps run by hand never open a batch. On a machine the app commands, batches open with Start cycle (sections 6.4 and 7).

# 3. Bluetooth setup on the ESP32

| Setting | Value |
| --- | --- |
| Role | BLE peripheral (GATT server), advertising so the phone can find it. |
| Device name | Use a clear name such as QualiHive. The app lists devices by name, or by address if there is no name. |
| Service UUID | 6e400001-b5a3-f393-e0a9-e50e24dcca9e |
| TX characteristic (ESP32 to phone) | 6e400003-b5a3-f393-e0a9-e50e24dcca9e Property: Notify. Add the 0x2902 descriptor (CCCD) so the phone can turn notifications on. All readings are sent here. |
| RX characteristic (phone to ESP32) | 6e400002-b5a3-f393-e0a9-e50e24dcca9e Property: Write, Write Without Response, or both. Required for pump control. When Write (with response) is offered, the app uses it, so every piece of a command is acknowledged. |
| Security | None. No pairing, bonding or PIN. |
| MTU | The app asks for an MTU of 185 bytes. If the ESP32 accepts it, one notification or write can carry up to 182 bytes. If not, the default is 20 bytes. Both work, because both sides split messages and join them again at the newline (sections 5.4 and 6.3). |
| After disconnect | Switch every pump off, then start advertising again so the phone can reconnect. |

Advertising the service UUID is recommended but not required. The app scans for every nearby BLE device and does not filter by service.

## 3.1 If the firmware uses other UUIDs

- Readings: the app looks for the TX UUID above. If it is not there, it uses the first characteristic on any service that can notify or indicate.

- Commands: the app looks for the RX UUID above. If it is not there, it uses the first writable characteristic on the same service the readings come from. It never writes to other services.

- Nothing writable: the app still connects and shows readings, but the pump controls are disabled. Home shows "This machine does not accept commands, so its pumps cannot be run from the app."

- Nothing that notifies: the connection fails with "No notify characteristic found on this device."

# 4. How the app connects

- The user opens More → Machine connection, keeps Bluetooth device selected and taps Scan for devices. Bluetooth must be on.

- Android asks for permission once: Nearby devices on Android 12 and newer, Location on Android 11 and older.

- The scan runs for 10 seconds and lists every BLE device it hears, by name or address.

- The user taps the machine in the Found list. The app connects (it gives up after 15 seconds), asks for the 185-byte MTU, finds the TX and RX characteristics, and turns on notifications on TX.

- The pill in the corner turns green. Home and Live update with every packet.

While connected:

- If no packet arrives for 12 seconds, the app treats the data as stale and stops showing the machine as online, even though the link is still open. Keep sending readings while the machine is idle.

- If the link drops, the app raises a Machine disconnected alert, ends any running pump cycle and marks every pump off.

- The app does not reconnect by itself. The user taps the machine again.

# 5. Sending readings (ESP32 to app)

## 5.1 Packet rules

- Send one JSON object per reading, as UTF-8 text.

- End every packet with a newline (\n). The app waits for the newline before it reads a packet. Without it, nothing shows up.

- Every field is optional, but each packet must carry at least one sensor value. A packet with only stage and machine_status is dropped.

- If a sensor is not ready or has failed, leave its field out. Do not send nan, inf or empty values. They make the JSON invalid and the app drops the whole line.

- Always send stage and machine_status. If machine_status is missing, the app assumes RUNNING and may start a batch.

- Key names are not case sensitive. Unknown keys are ignored, so you can add extra fields for debugging.

- Numbers can be sent as numbers (3.82) or as text ("3.82").

- Suggested rate: one packet per second, and never more than a few seconds apart (see the 12-second rule in section 4). Each packet is about 250 to 300 bytes.

## 5.2 Example packet

Shown on several lines here to make it easier to read. Send it as a single line followed by \n.

{

"type": "sensor_update",

"batch_id": "QH-2026-0084",

"ph": 3.82,

"temperature_c": 29.7,

"conductivity_ms_cm": 1.71,

"turbidity_ntu": 8.6,

"weight_kg": 1.83,

"flow_l_min": 0.80,

"color": { "r": 215, "g": 142, "b": 56, "classification": "Amber" },

"stage": "QUALITY_ASSESSMENT",

"machine_status": "RUNNING"

}

## 5.3 Fields

| Key | Type | Unit | Notes |
| --- | --- | --- | --- |
| type | text |  | Send "sensor_update" ("reading" and "sample" also work). A packet with any other type, for example "log" or "ack", is ignored. A packet with no type is treated as a reading. |
| batch_id | text |  | Optional. Groups readings into one batch, for example "QH-2026-0084". A new value closes the previous batch. If you leave it out, the app makes its own batch code. |
| ph | number | pH | Example: 3.82 |
| temperature_c | number | °C | Example: 29.7 |
| conductivity_ms_cm | number | mS/cm | Example: 1.71 |
| turbidity_ntu | number | NTU | Example: 8.6 |
| weight_kg | number | kg | Weight in the jar. Recorded but not graded. Fills the jar on the Home screen (1 kg = full). |
| flow_l_min | number | L/min | Only if a flow sensor is fitted. Recorded but not graded. Sets the speed of the flow animation on Home. |
| color | object |  | r, g, b as whole numbers from 0 to 255 (values outside are clamped). Optional classification: Water white, Extra white, White, Extra light amber, Light amber, Amber or Dark amber. If the ESP32 computes a Pfund value, send "pfund": 68.0 inside this object. A bare number ("color": 68.0) is read as a Pfund value. |
| stage | text |  | Where the batch is in the filtration sequence. See 5.5. |
| machine_status | text |  | Overall run state of the machine. See 5.5 and 6.4. |

Short names from the early prototype (temp, ec, ntu, weight, flow) are still accepted, but please use the names in the table. Moisture is no longer measured: moisture_percent and moisture are ignored if sent.

## 5.4 Splitting long packets

A packet is usually longer than one BLE notification. Split it into chunks and send them one after another on the TX characteristic. The app joins the chunks and cuts them at each \n. You do not need to add any header or length.

- 20 bytes per notification always works. Up to 182 bytes works if the larger MTU was agreed.

- Leave a short delay (about 10 ms) between notifications so the ESP32 Bluetooth stack does not drop any.

- If the app receives 4096 bytes without a newline, it throws that data away.

## 5.5 Stage and machine status values

Send these as uppercase text. The app also accepts lowercase and some common alternatives, but these are the standard values. An unknown stage counts as IDLE; an unknown machine_status counts as RUNNING.

| stage | Meaning | In the app's pump cycle |
| --- | --- | --- |
| IDLE | Machine idle. No batch is running. | No pump running |
| EXTRACTING | Drawing honey into the system. The app starts a batch here. | Pump 1 |
| PRIMARY_FILTRATION | Coarse filtration. | Pump 2 |
| SECONDARY_FILTRATION | Fine filtration. | Pump 3 |
| QUALITY_ASSESSMENT | Measuring quality. The final grade is the average of the readings sent during this stage, so send several readings here. | Pump 4 |
| FINAL_TRANSFER | Transferring the filtered honey. | Not used |
| COMPLETED | Batch finished. The app closes and grades the batch. | Do not send (see 6.4) |
| PAUSED | Sequence paused. |  |
| ERROR | Sequence stopped by a fault. |  |

| machine_status | Meaning |
| --- | --- |
| READY | Ready to start a batch. Send this whenever no pump is running. |
| RUNNING | Filtration in progress. Send this while any pump is running. |
| PAUSED | Cycle paused. |
| COMPLETED | Cycle complete. This also closes the batch. |
| ERROR | The machine has a fault. Home shows a fault on the machine picture. |

Accepted alternatives: IDLE, STANDBY, WAITING mean READY; RUN, ACTIVE, BUSY mean RUNNING; DONE, FINISHED, COMPLETE mean COMPLETED; HALTED, STOPPED, SUSPENDED mean PAUSED; FAULT, ERR, FAILURE mean ERROR.

# 6. Pump control (app to ESP32)

## 6.1 The four pumps

The machine moves honey in four hops, each driven by its own pump. Each pump is switched on and off on its own, and several can run at the same time. The app runs them in two ways: a timed cycle (pump 1, then 2, then 3, then 4, each for its own time), and a switch for each pump that the user can turn on or off at any time, even in the middle of a cycle.

| Pump | Moves honey | Readings filed under stage |
| --- | --- | --- |
| 1 | Hopper → Filter 1 | EXTRACTING |
| 2 | Filter 1 → Filter 2 | PRIMARY_FILTRATION |
| 3 | Filter 2 → Filter 3 | SECONDARY_FILTRATION |
| 4 | Filter 3 → Jar | QUALITY_ASSESSMENT (the batch grade comes from these readings) |

## 6.2 Commands

The app writes these to the RX characteristic. Each command is one JSON object on one line, ending in \n:

{"type":"pump_on","pump":2,"duration_s":90}

{"type":"pump_off","pump":2}

{"type":"pump_off"}

| Key | Type | Notes |
| --- | --- | --- |
| type | text | "pump_on" or "pump_off". |
| pump | whole number | pump_on: required, 1, 2, 3 or 4. pump_off: optional. With it, only that pump goes off; without it, every pump does. |
| duration_s | whole number | pump_on only. How many seconds the pump runs: 1 to 7200 (2 hours). The ESP32 must switch that pump off by itself when this runs out. |

| Command | What the ESP32 must do |
| --- | --- |
| pump_on | Switch this pump on and start (or restart) its own timer for duration_s seconds. Leave every other pump as it is. When the timer runs out, switch this pump off, even if no other command has arrived. |
| pump_off | With pump: switch that pump off. Without pump: switch every pump off. Switching off a pump that is already off is not an error: do nothing. |

A command never touches a pump it does not name: pump_on for pump 3 leaves pumps 1, 2 and 4 exactly as they were. There is no separate start or stop for the cycle: a cycle is a series of pump_off and pump_on commands for single pumps (section 6.5).

## 6.3 Rules for the firmware

- Buffer until the newline. Collect the bytes written to RX and act on each complete line when the \n arrives. The app splits a command into pieces of the MTU minus 3 bytes: 20 bytes at the default MTU, so a pump_on (about 45 bytes) arrives in three writes. Acting on each write by itself breaks the command.

- Each pump on its own. Keep a separate on/off state and a separate timer for each pump. A pump_on, or a pump_off with a pump, changes only that pump.

- Start with every pump off at power-up and after a reset.

- Ignore what you do not understand. Skip lines that are not valid JSON and command types you do not know. If a pump_on has a pump that is not 1 to 4, or duration_s is missing or zero, switch everything off and do not start a pump. A pump_off whose pump is not 1 to 4 also switches everything off.

- Answer each command with a fresh reading (with sensor values, see 5.1) so the app shows the change straight away. The in-app simulator does the same.

- Commands need no reply message. If RX offers Write with response, the BLE acknowledgement is all the app waits for.

Safety: the ESP32, not the phone, makes sure a pump stops. Always honour duration_s, for each pump separately and including pumps switched on by hand: the phone sends the next command when the time is up, but if the phone sleeps, the app is closed or the link drops, nothing more will arrive. And when the phone disconnects, switch every pump off.

## 6.4 Readings while the app runs the pumps

- Send machine_status RUNNING while any pump is running, and READY when none is.

- For stage, send the stage of the highest-numbered pump that is running (table in 6.1), or IDLE when none is. If the packet says IDLE, or has no stage, while the app has a pump on, the app files the reading under that same stage itself. A stage you do send is kept as it is.

- Do not send COMPLETED while the app is running the cycle. The app closes the batch itself after pump 4. A COMPLETED in the middle closes the batch early, and the next RUNNING packet opens a second one.

- After the cycle, send READY whenever no pump is running. A pump running by hand is reported as RUNNING like any other: on a machine the app commands, readings outside a cycle never open a batch (unless they carry a batch_id). If a batch is already open, they join it.

- Send several readings while pump 4 runs. The batch grade is the average of the readings taken then.

## 6.5 What the app sends, and when

| When | The app sends |
| --- | --- |
| The user taps Start cycle on Home | pump_on for pump 1, with pump 1's time. The batch opens once this write has gone through. |
| Pump 1's time is up, or the user taps Skip | pump_off for pump 1, then pump_on for pump 2 with pump 2's time. The same happens for pumps 3 and 4. Pumps the cycle is not on are not mentioned. |
| Pump 4's time is up, or Skip on pump 4 | pump_off for pump 4. The cycle is complete and the batch closes and is graded. Pumps running by hand stay on. |
| The user taps Stop (during a cycle) or All off (with no cycle) | pump_off with no pump: every pump goes off, including pumps running by hand. The batch stays open until the user taps Finish. |
| The user switches a pump on from Home | pump_on for that pump with duration_s 7200 (2 hours). If it is the pump the cycle is on, it gets only the seconds left in that step. |
| The user switches a pump off from Home | pump_off for that pump. A running cycle carries on: when that pump's step ends, the app still sends pump_off for it and pump_on for the next. |
| The user taps Finish batch during a cycle | pump_off, then the batch closes. |
| The user taps Disconnect while any pump is on | pump_off, then the app disconnects. |
| The user switches the source to Simulator while any pump is on | pump_off over the Bluetooth link, then closes it. |
| A cycle command could not be written | One more pump_off, if it can. The cycle stops and Home shows "Could not reach the machine: …". A switch whose command fails stays where it was, the app shows "Could not switch pump N on: …" and the cycle carries on. |
| The link drops | Nothing, because it cannot. The ESP32's timers and disconnect rule stop the pumps. The app marks every pump off and Home shows "The link dropped during pump N. Reconnect and start again." |

- The app moves to the next pump only after the write for it has gone through.

- The pump_off and the next pump_on are sent when the phone's timer runs out, at about the same moment the ESP32's own timer for that pump runs out, so a pump_off for a pump that is already off is normal. A short gap between one pump stopping and the next starting is normal too.

- Each pump gets its full time from when its command is sent. If the phone was busy or asleep, the next command can come late: the cycle then takes longer, but no pump runs longer than its time.

- Switches never move the cycle's clock. If the user switches the cycle's pump off, the countdown carries on; switched back on, the pump runs for what is left of its step.

- When the cycle reaches a pump that is already on by hand, it sends pump_on for it again with the pump's set time. Restart that pump's timer with the new duration_s.

## 6.6 Pump timers in the app

- Set in More → Pump timers. Each pump has its own time.

- Default: 1 minute per pump. Allowed: 5 seconds to 2 hours. Presets: 30 s, 1, 2, 5 and 10 minutes, or type minutes and seconds.

- The screen shows the time for the full cycle. Home shows it too, under Start cycle.

- Times are saved on the phone. A cycle keeps the times it started with, so a change applies from the next cycle.

- The restore button at the top resets every pump to the default.

- These times are for the cycle only. A pump switched on by hand runs until it is switched off, for at most 2 hours.

## 6.7 Example sketch (ESP32 Arduino)

A complete minimal sketch for the ESP32 Arduino core, using its built-in BLE library and ArduinoJson 7. It follows every rule in this guide. The pump pins and the read…() sensor functions are placeholders: replace them with your wiring and sensor code.

#include <BLEDevice.h>

#include <BLEServer.h>

#include <BLE2902.h>

#include <ArduinoJson.h>   // ArduinoJson 7

#define SERVICE_UUID "6e400001-b5a3-f393-e0a9-e50e24dcca9e"

#define RX_UUID      "6e400002-b5a3-f393-e0a9-e50e24dcca9e"   // phone -> ESP32

#define TX_UUID      "6e400003-b5a3-f393-e0a9-e50e24dcca9e"   // ESP32 -> phone

// Pump 1..4. Change to your wiring. Swap HIGH/LOW for relays that switch on LOW.

const int PUMP_PINS[4] = {25, 26, 27, 14};

const char* STAGE_FOR_PUMP[5] = {"IDLE", "EXTRACTING", "PRIMARY_FILTRATION",

"SECONDARY_FILTRATION", "QUALITY_ASSESSMENT"};

BLECharacteristic* tx;

volatile bool connected = false;

volatile bool sendNow = false;

volatile bool pumpIsOn[5] = {false};  // index 1..4; [0] unused

volatile unsigned long pumpOffAt[5];  // each pump has its own timer

String rxBuffer;

unsigned long lastSent = 0;

void pumpOff(int pump) {

digitalWrite(PUMP_PINS[pump - 1], LOW);

pumpIsOn[pump] = false;

}

void allPumpsOff() {

for (int p = 1; p <= 4; p++) pumpOff(p);

}

void pumpOn(int pump, unsigned long seconds) {

digitalWrite(PUMP_PINS[pump - 1], HIGH);   // the other pumps stay as they are

pumpIsOn[pump] = true;

pumpOffAt[pump] = millis() + seconds * 1000UL;   // a repeat pump_on restarts it

}

int highestPumpOn() {                   // 0 = none

for (int p = 4; p >= 1; p--) if (pumpIsOn[p]) return p;

return 0;

}

void handleCommand(const String& line) {

JsonDocument doc;

if (deserializeJson(doc, line)) return;          // not JSON: ignore

const char* type = doc["type"] | "";

int pump = doc["pump"] | 0;

bool validPump = pump >= 1 && pump <= 4;

if (strcmp(type, "pump_off") == 0) {

if (validPump) pumpOff(pump);                  // just this pump

else allPumpsOff();                            // no pump: every pump

} else if (strcmp(type, "pump_on") == 0) {

unsigned long seconds = doc["duration_s"].as<unsigned long>();

if (validPump && seconds > 0) pumpOn(pump, seconds);

else allPumpsOff();

} else {

return;                                         // unknown type: ignore

}

sendNow = true;                     // answer with a fresh reading

}

class RxCallbacks : public BLECharacteristicCallbacks {

void onWrite(BLECharacteristic* c) {

rxBuffer += String(c->getValue().c_str());

int nl;

while ((nl = rxBuffer.indexOf('\n')) >= 0) {   // a command may come in pieces

String line = rxBuffer.substring(0, nl);

rxBuffer.remove(0, nl + 1);

handleCommand(line);

}

if (rxBuffer.length() > 512) rxBuffer = "";   // never got a newline

}

};

class ServerCallbacks : public BLEServerCallbacks {

void onConnect(BLEServer* server) { connected = true; }

void onDisconnect(BLEServer* server) {

connected = false;

allPumpsOff();                      // no phone, no pumping

rxBuffer = "";

server->getAdvertising()->start();  // let the phone reconnect

}

};

void sendLine(String line) {

line += "\n";

const unsigned int CHUNK = 20;      // works at any MTU

for (unsigned int i = 0; i < line.length(); i += CHUNK) {

String part = line.substring(i, i + CHUNK);

tx->setValue((uint8_t*)part.c_str(), part.length());

tx->notify();

delay(10);

}

}

void sendReading() {

JsonDocument doc;

doc["type"] = "sensor_update";

// Your sensor code. Leave out any value that is not ready (never send NaN).

float ph = readPh();

if (!isnan(ph)) doc["ph"] = ph;

doc["temperature_c"] = readTemperatureC();

doc["conductivity_ms_cm"] = readConductivity();

doc["turbidity_ntu"] = readTurbidity();

doc["weight_kg"] = readWeightKg();

JsonObject color = doc["color"].to<JsonObject>();

color["r"] = readRed();  color["g"] = readGreen();  color["b"] = readBlue();

int top = highestPumpOn();          // the honey furthest along

doc["stage"] = STAGE_FOR_PUMP[top];

doc["machine_status"] = top ? "RUNNING" : "READY";

String line;

serializeJson(doc, line);

sendLine(line);

}

void setup() {

for (int i = 0; i < 4; i++) {

pinMode(PUMP_PINS[i], OUTPUT);

digitalWrite(PUMP_PINS[i], LOW);    // every pump off at power-up

}

BLEDevice::init("QualiHive");

BLEDevice::setMTU(185);

BLEServer* server = BLEDevice::createServer();

server->setCallbacks(new ServerCallbacks());

BLEService* service = server->createService(SERVICE_UUID);

tx = service->createCharacteristic(TX_UUID, BLECharacteristic::PROPERTY_NOTIFY);

tx->addDescriptor(new BLE2902());

BLECharacteristic* rx = service->createCharacteristic(RX_UUID,

BLECharacteristic::PROPERTY_WRITE | BLECharacteristic::PROPERTY_WRITE_NR);

rx->setCallbacks(new RxCallbacks());

service->start();

server->getAdvertising()->start();

}

void loop() {

// The safety backstop: each pump stops when its own time is up, phone or no phone.

for (int p = 1; p <= 4; p++) {

if (pumpIsOn[p] && (long)(millis() - pumpOffAt[p]) >= 0) {

pumpOff(p);

sendNow = true;

}

}

if (connected && (sendNow || millis() - lastSent >= 1000)) {

sendNow = false;

lastSent = millis();

sendReading();

}

delay(10);

}

# 7. Batches and grading

What the app does with the readings, so the firmware sends what it needs.

| How the machine is run | When a batch opens and closes |
| --- | --- |
| App runs the pumps (normal case) | Opens when pump 1 is confirmed running. Closes after pump 4. Stop leaves it open; Finish on Home closes it. Pumps switched on by hand never open a batch; while one is open, their readings join it. |
| Machine runs its own sequence (takes no commands, reports stages) | Opens when stage leaves IDLE or machine_status is RUNNING. Closes on COMPLETED. |
| Machine takes no commands and reports no stage | The user taps Start on the batch card on Home, and Finish to close it. |
| ESP32 sends batch_id | The ESP32 decides. A new value closes the previous batch and opens the new one. |

- Grade: the average of the readings taken during QUALITY_ASSESSMENT (pump 4). If there are none, the average of all the batch's readings. Weight is the last value received.

- Result: every parameter is Acceptable, Warning, Outside range or No data. The batch is ACCEPTABLE, REQUIRES ATTENTION, OUTSIDE SELECTED QUALITY PARAMETERS or INCOMPLETE, with a recommendation. A batch is never acceptable while a graded sensor is silent.

- Alerts: batch started and completed; a parameter reaching Warning or Outside range (once per level, again if it gets worse); a sensor that reported earlier in the batch stopping; and the machine disconnecting.

- The verdict is frozen onto the batch when it closes, together with the reference values used.

Default reference values. They can be changed in More → Quality reference values.

| Parameter | Accepted | Warning up to | Note |
| --- | --- | --- | --- |
| pH | 3.7 – 4.0 | 3.5 – 4.2 |  |
| Conductivity | 1.28 – 2.52 mS/cm | 1.10 – 2.80 mS/cm |  |
| Colour | 34 – 150 mm Pfund | from 25 mm | RGB calibration provisional |
| Temperature | ≤ 40 °C | ≤ 45 °C | Process limit |
| Turbidity | ≤ 10 NTU | ≤ 25 NTU | Provisional |
| Weight, flow | Not graded |  | Recorded only |

# 8. Testing without the machine

- In-app simulator. In More → Machine connection, switch the source to Simulator. It behaves the way the firmware should: idle until the app runs a pump, each pump switched on and off on its own and switching itself off when its time is up, and a fresh reading after each command. Every third lot has a defect, so every result shows up.

- PC device simulator (device-simulator/run.bat in the repository). Turns a Windows PC with a Bluetooth adapter into a BLE device with the same service. It sends readings from sliders, can step through the stages, and sends any JSON line you type. It has no RX characteristic yet, so the app treats it as a machine that takes no commands: the pump controls are disabled and Start on the batch card opens a batch by hand.

- nRF Connect (free, Android and iOS) on a second phone. Connect to the ESP32 and turn on notifications on TX to watch the readings. To test a command, write it to RX as bytes, ending in 0A (the newline).

- Serial Monitor. While bench testing, pass lines from the Serial Monitor (line ending set to Newline) to the same command handler, to try the pumps without a phone.

# 9. Troubleshooting

| Problem | What to check |
| --- | --- |
| The ESP32 is not in the Found list | Is it advertising, and does it start again after a disconnect? Is Bluetooth on, and was the Nearby devices (or Location) permission allowed? |
| "No notify characteristic found on this device." | TX needs the Notify property. |
| Connected, but nothing shows on Home or Live | Each packet needs a \n at the end, valid JSON (no nan), a type of "sensor_update" or none, and at least one sensor value. TX needs the 0x2902 descriptor. |
| The machine shows as offline while connected | No packet for 12 seconds. Keep sending about once a second, even when idle. |
| Start cycle is greyed out: "This machine does not accept commands" | RX is missing, or is not writable, or is on a different service from TX. |
| The cycle stops with "Could not reach the machine: …" | A write failed. Check that RX accepts writes and that the link is stable. |
| The pumps do not respond | Is the firmware buffering until the \n? A command arrives in several pieces at the default MTU. |
| A pump keeps running after the phone leaves | Fix before use. The firmware must switch the pump off when duration_s runs out and when the phone disconnects. |
| Switching one pump changes another one too | The firmware still follows the October 29 guide. pump_on must leave the other pumps alone, and pump_off with a pump must switch off only that one. |
| The batch closes in the middle of a cycle | The ESP32 sent COMPLETED, or a new batch_id. |
| The batch result is INCOMPLETE | A graded sensor never reported, or stopped reporting, during the batch. Check the alerts. |
| Packets arrive garbled or some are missing | Add about 10 ms between notifications, and keep each chunk within the MTU. |

# 10. Firmware checklist

- Nordic UART Service with TX (Notify + 0x2902) and RX (Write and/or Write Without Response), advertised under a clear name, and again after every disconnect.

- One JSON reading per line, ending in \n, about once a second, split into chunks of 20 bytes (or MTU minus 3) with about 10 ms between them.

- Every reading has at least one sensor value, plus stage and machine_status. No nan or inf, and no moisture_percent.

- machine_status is RUNNING while any pump runs and READY otherwise, and stage is that of the highest-numbered running pump. No COMPLETED during an app-run cycle.

- RX bytes are buffered and handled one line at a time at the \n.

- pump_on switches on only the pump it names, leaves the others alone, and switches that pump off by itself after its own duration_s.

- pump_off with a pump switches only that pump off; without one, every pump.

- Every pump goes off on disconnect and at power-up.

- A fresh reading goes out after each command.